KMU 501 Advanced Chemical Engineering Thermodynamics

Fall Semester, Homework IV

Instructor: Selis Önel, PhD

Question 1:	Determine the equation of state parameters (virial coefficients) a and b for the van der Waals equation given by  .    Show all your work.

Hint: Use the critical point and the Tc isotherm for the P-V graph

Question 2:	Consider two tanks each having a volume of 1 dm3. One of the tanks is filled with saturated liquid water and the other is filled with saturated vapor (steam), both at a pressure of 10 bar. If both tanks explode, which will do the greater damage?

Hints:

- Assume that the tanks expand rapidly to a pressure of 1 bar in an adiabatic process. Damage will be proportional to the work done by the system.

- As the answer to the question is comparative, assume the adiabatic expansion is reversible, i.e. it has constant entropy, and produces the maximum possible work or damage.

- Use the Steam Tables for saturated liquid and saturated vapor data

(Sandler Appendix III p.584; Smith, van Ness and Abbott, Table F.1, p.716; or other source)

- Show the calculations for both cases

Question 3:

For an ideal gas with constant heat capacities (Cv and Cp)    and   integrate to give    and   , where    and   are integration constants.

Show that a third equation of this kind is    .

Question 4:

Derive equations relating T and V, T and P, and P and V for the reversible, adiabatic compression or expansion of an ideal gas with constant heat capacities.

Hint: For a reversible process    and for an adiabatic process  .

Question 5:

Show that for an ideal gas with constant heat capacities, the slope of a PV curve for a reversible, adiabatic process is negative and that it has a larger absolute value than the slope of a PV curve for an isotherm at the same values of P and V.

Hint: You may use the relations between P and V for a reversible and adiabatic process.